Concrete Beam Utilities

Program Description:

CBM is used to input information and is called by the analysis programs.  If the user chooses not to input new information, analysis is run on existing values.  This  is useful when a user has already input information and then only wants to change one variable (say area of steel) which can be done by hand before running the program.

BETA calculates the beta factor based on concrete strength.

VCAP calculates the ultimate shear capacity of a beam section given values stored in fc, d, b, fy, Av (area of steel per stirrup), and s (stirrup spacing).  The strength reduction factor of f = 0.85 is included.  Checks are made for reduced allowable stirrup spacing and maximum allowable shear resistance of stirrups based on concrete strength (AASHTO Sec. 8.)

Program:  CBM

%%HP: T(3)A(D)F(.);

\<< { { "YES"

\<< 1 SF CONT

\>> } { "NO"

\<< 1 CF CONT

\>> } } TMENU

"Input new data?"

PROMPT

IF 1 FS?

THEN { { "CONT"

\<< CONT

\>> } } TMENU

"b (in)?" PROMPT

'b' STO "d (in)?"

PROMPT 'd' STO

"f'c (psi)" PROMPT

'fc' STO

"Area of steel (in^2)?"

PROMPT 'As' STO

"fy (psi)?" PROMPT

'fy' STO

IF 2 FS?

THEN

"d' (in)?

Dist to compr. steel"

PROMPT 'dp' STO

"As' (in^2)?

Area compression steel"

PROMPT 'Asp' STO

ELSE

IF 3 FS?

THEN

"bw (in)?" PROMPT

'bw' STO "hf (in)?"

PROMPT 'hf' STO

END

END

END BETA 1 CF 1

MENU

\>>

# 7079h

534.5 bytes

Program:  BETA

%%HP: T(3)A(D)F(.);

\<<

IF 'fc\>=4000'

THEN

\<<

IF 'fc\>=8000'

THEN .65

ELSE fc 4000

- 1000 / .05 * NEG

.85 +

END

\>>

ELSE .85

END '\Gb1' STO

\>>

# DB69h

202 bytes

Program:  VCAP

%%HP: T(3)A(D)F(.);

\<< fc \v/ 2 * b * d *

'Vc' STO Av fy * d

* s / 'Vs' STO

IF s d 2 / > s 12

> OR Vs 2 Vc * >

AND

THEN

"Stirrups too far apart"

DOERR

END

IF Vs Vc 4 * >

THEN "Vs reduced"

Vc 4 * 'Vs' STO

CLLCD 1 DISP 1

FREEZE

END Vc Vs + .85 *

1000 / "Vu (kips)"

\->TAG

\>>

# 94h

336.5 bytes